Highways Legal Agreements

Section 278 Drawing Submission Checklist

Document Version Control

| Rev. | Status | Author | Approver | Date |
| --- | --- | --- | --- | --- |
| 0 | Approved | AH | HA | 19/02/26 |

Introduction

This Section 278 Drawing Submission Checklist is to be read in conjunction with the Application for the adoption of roads pursuant to Section 278 of the Highways Act 1980 document.

Planning Consent

The applicant must have planning consent for the works prior to application. GCC’s comments primarily focus on adoptability concerns and safety issues. Other suggestions may be provided, but it is expected that all designs proposed to GCC for adoption will have been designed in accordance with both local and national standards by a competent designer as required by the Construction (Design and Management) Regulation 2015.

How to submit

To make a Section 278 application:

- Submit a completed Section 278 Application Form to highwaylegalagreements@gloucestershire.gov.uk.

- The team will provide a unique reference number to track the application throughout.

- GCC will provide instructions outlining how to make the full submission. At this point, you must submit a completed version of the Section 278 Submission Checklist along with all the required information.

- An application will only be progressed when the initial fee is paid - provided all other supporting documents have been submitted. Refer to Section C of the application form for more information. Until this fee is paid, the application will not be considered for Technical Approval.

Checklist

A checklist has been provided to help guide the applicant regarding the documents and drawings that are required to be submitted for this type of Design Audit.

Table 1 below is a checklist that shows the plans and information GCC requires to be submitted.

Table 2 provides guidance on the information that is expected to be shown within each of the required documents.

Only once all elements have been satisfied will the submission be progressed to the Design Audit Stage.

This document should be read in conjunction with the current revision of Gloucestershire County Council’s Technical Specification for New Streets document.

| Drawing Name | Drawing Number | Received by GCC (to be filled in by GCC officer) |
| --- | --- | --- |
| MANDATORY |  |  |
| Site Location Plan(s) |  |  |
| Existing Site Plan(s) |  |  |
| General Arrangement Plan(s) |  |  |
| Longitudinal Sections Plan(s) |  |  |
| Construction Details Plan (s) |  |  |
| Signing & Lining Plan (s) |  |  |
| Kerbing & Surfacing Plan (s) |  |  |
| Section 278 Legal Agreement Plan |  |  |
| Existing & Proposed Utilities Plan(s) & Information |  |  |
| Site Clearance Plan(s) |  |  |
| Gully Catchment Plan(s) |  |  |
| Engineering Plan(s) |  |  |
| Vehicular Swept Path Plan(s) |  |  |
| Street Lighting Submission |  |  |
| Highway Drainage Submission (Not required if offering gullies/connection pipes ONLY for adoption) |  |  |
| Any other relevant engineering drawings not listed above: |  |  |
| SUPPORTING DOCUMENTS |  |  |
| Land Registry Title documents and plan |  |  |
| Correspondence evidencing that the non-refundable initial fee has been requested and subsequently paid to GCC. See “How to submit” section of this document. |  |  |
| Planning decision notice |  |  |
| Road Safety Audit Brief (Stage 2 or combined 1/2) in line with GG119 |  |  |
| Copy of relevant Section 106 agreement |  |  |
| Construction Management Plan |  |  |
| Any other relevant supporting information |  |  |

Table 2 - Section 278 Submission Checklist Guidance Notes

Site Location Plan

- Ordinance Survey Data used for submission must be dated within 5 years of submission date, with dates and companies clearly shown on the drawing - Scale must be either 1:1250 or 1:2500 - North point must be clearly shown - Application site must be edged in red - Surrounding road names must be shown

Existing site plan

- Topographical Survey Data used for submission must be dated within 5 years of submission date, with dates and companies clearly shown on the drawing - Drawing to be shown with existing colours/key

General Arrangement Plan

- Show the dimensions of all roads, footways and verges - Road identification: Road 1, 2 etc. - Chainage points for the start/end, tangent points and ever 10m - Forward visibility, Junction visibility, pedestrian visibility (different colours for each design speed) - All junction radii in adoptable areas (shown as different colours in key)

Longitudinal Sections Drawing

- Existing and proposed levels for the centreline, chainage, channels, gradients and vertical curves with K values, horizontal schematic showing curve radii and transitions - Existing/proposed Highway drainage to be coloured purple - Surface and foul water drainage profiles, positions of chambers, gradients, pipe diameters, cover and invert levels and protection - Existing ground level (based on submitted survey data) shown as a dashed green line - All roads proposed for adoption must have gradients between 1/20 and 1/100 - GCC may request vertical visibility sections (if necessary)

Construction Details Drawing

Details are required for each adoptable element within the project unless reference is made to follow GCC’s standard details. For example: - Typical cross sections of carriageway, footway, cycleway, ramps etc. - Construction details indicating areas for different pavement types, planning depths, layer thicknesses and areas where special materials such as geotextiles are to be used - Kerbing details - Pedestrian crossing with tactiles details - Tie-in detail - Note that this is not an exhaustive list and other details will be required.

Signing & Lining Drawing

- Cross section of bollard and pole foundations - Traffic sign and road marking information including areas of high friction surfacing - Sign schedules must be produced for each sign giving post sizes/foundations, x heights/dimensions, mounting height, illumination and retro-reflectivity details as a minimum - Road Marking schedule must be produced, showing each marking type and all relevant details - Must include Structural details of proposed sign foundations for any non-standard sign foundations

Kerbing & Surfacing Drawing

- Show all kerbing and surfacing types proposed for the site in the drawing key - All items in the key should reference the construction details and have a detail provided - Where possible, areas of parking bays resulting in >10m dropped kerb should have their footway crossfalls locally amended to a 1/40 crossfall gradient to benefit pedestrian movements - Illustration of kerb types should be coloured lines and clear key

Section 278 Legal Agreement Plan

- Coloured in accordance with GCC Legal Agreement Plan key - Correct highway boundary shown as obtained from GCC Highway Records department - Adjacent land titles held by the developer(s) must be shown edged in blue lines

Existing & Proposed Utilities Plan(s) & Information

- Evidence that the Statutory Undertakers have been notified of the works and any apparatus within the scheme is known and diversionary/protection works are being actively progressed. - Plan to show all existing utilities within and surrounding the site

Site Clearance Plan

- Show all existing and show all proposed to be removed/ relocated in separate colours.

Gully Catchment Plans

- GCC will accept a maximum of 150m2 to a single gully and 200m2 to double gullies - Double gullies are required at all low spots - Gullies must be installed upstream of all uncontrolled crossing locations, unless it is evidenced that SW flows will not pass in front of the uncontrolled crossing. For example, gully is located at a high point.

Engineering Plan

- Road contours must be shown at 50mm increments as dashed lines - Every 0.5m increment should be shown as a solid line - Drawing to show any proposed embankments/retaining features - Mains drainage details must be shown

Vehicular Swept Path Plan

- The plans should match what was submitted for planning - Must show largest vehicles traversing the network.

Street Lighting Submission

- GCC require submission of the following information: - Street lighting plan - Outdoor lighting report - Submissions package to include: Street lighting calculations, proposed equipment, levels, planting details, driveway details. - Note that all street lighting needs to be on Highway or on proposed Highway - This should also include all bollards (lit and retro-reflective) and internally and externally lit traffic signs

Highway Drainage Plan (Not required if offering gullies/connection pipes ONLY for adoption)

- Show all SW drainage (proposed and existing) within the vicinity of the site that connect with highway drainage in black for background context (details not required) - Layout, showing existing and proposed: pipe runs (with sizes, gradients, diameters, depths, cover, backfill), manhole and gully, sizes, inverts positions and details, outfall details, backfill details, unique numbering of pipes / gullies and chambers. - Sewers must be positioned a minimum of 1m, and manholes are 0.5m from outside edge of the kerb line. - Show Manholes and attenuation device chambers, Storage chambers, Headwalls proposed for adoption - Show the catchment in m2 going to each manhole (outline only, hatch not required) - All highway drainage to be coloured purple

Roundabouts (if present)

- Plan to show proposed levels, contours and crown lines - Checklist to ensure compliance with CD 116 Geometric Design of Roundabouts - Provide existing, generated and forecast turning flows for the morning and evening peak periods - Provide capacity calculations for the AM and PM peak periods using ARCADY or similar - Plan showing entry path deflection, entry and exit radius, entry and exit widths and flare lengths - Plan showing visibility lines - Plan to show pedestrian and cycling facilities through the junction